Vicinity Permit No. 	:

Vicinity Permit User	: Wanto ( MKN ) / Z40363

Date	:

| AREA | LINES | ELECTRICIAN SAFE DISTANCE |  |  | NON ELECTRICAL / EQUITMENT VICINITY SAFE DISTANCE |  |  |  | PPE & TOOL | PERMIT | ISOLATED LOCKOUT | ELECTRICAL SUPERVISION | REFERENSI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Voltage | Open Conductor | Isolated Conductor | Kondisi | s/d 400 V | s/d 20 KV | s/d 70 KV |  |  |  |  |  |
| ⎕Tanjung Bara ⎕Batu Putih ⎕Swarga Bara ⎕Overland Conv. ⎕CPP – ROM ⎕ROM – Melawan ⎕Bengalon | ⎕Distribution ⎕Transmission | ⎕380 V ⎕3.300 V ⎕11.000V ⎕20.000V ⎕70.000 V | 50 cm 60 cm 75 cm 204 cm 250 cm | 30 cm 50 cm 60 cm 80 cm 100 cm | ⎕Jaringan menyeberang jalan raya ⎕Jaringan ke tanah ⎕Jaringan ke ujung pohon ⎕Jaringan dekat jalan haul batubara ⎕Jaringan sepanjang jalan raya | 6 M 5 M 1.5 M 5 M | 6.5 M 6.5 M 3 M 6.5 M | 8 M 8 M 5 M 8 M | ⎕Helmet ⎕Safety glass ⎕Masker ⎕Ear plug ⎕Gloves ⎕Safety Shoes ⎕Harness ⎕Full harness ⎕Wear pack ⎕Reflector ⎕Special HV tools ⎕Bucket Truck | ⎕Vicinity ⎕Access | ⎕Harus ⎕Tidak perlu | Nama & Tanda tangan 1. 2. 3. | a. PUIL 2000 b. Diklat PLN c. Australian Std. d. KPC Std. e. IEC 62271 |
| catatan : Beri tanda √ pada kotak sesuai keperluan keterangan : - Vicinity Permit Issuer - ________________________________________________________ - ________________________________________________________ - ________________________________________________________ - ________________________________________________________ - ________________________________________________________ ( Supardi/16766 ) |  |  |  |  |  |  |  |  |  |  |  |  |  |